AI 407 – Introduction to Robotics

Lab 7 Manual

OpenManipulator-X Programming and Control – Part 2

- Lab objectives

- Learn to use MoveIt for motion planning and robot control.

- Visualizing robot movements and sensor data in RViz.

- Learn how to subscribe to the /joint_states topic.

- Lab Requirements

- Software: Ubuntu 22.04 LTS, ROS 2 Humble

- Hardware: Students should work on Lab Devices

- Before You Start

Kindly read the manual, review the references if any, before beginning implementation.

- Introduction to Moveit & RViz

MoveIt is a powerful motion planning framework in ROS that allows you to control robotic arms and other robotic manipulators. It provides the following key features:

- Motion Planning: Calculates a sequence of movements from the robot's current to a goal position.

- Collision Checking: Ensures that the robot does not collide with its environment during movement

- Inverse Kinematics (IK): Solves for the joint configurations required to achieve a desired pose.

- Execution: Executes the motion plan by controlling the robot's actuators.

MoveIt is responsible for motion planning, controlling the manipulator, and handling tasks such as path planning, inverse kinematics (IK), and trajectory execution.

RViz is a visualization tool that provides a 3D interface where you can interact with the robot, visualize its movements, and plan trajectories using the MoveIt plugin for RViz.

In the following table we can see the key differences between Gazebo, Moveit, and RViz tools:

| Feature | Gazebo | MoveIt | RViz |
| --- | --- | --- | --- |
| Purpose | Robot simulation (physics and environment) | Motion planning and manipulation | Robot data visualization and interaction |
| Core Focus | Physics, environment, robot simulation | Path planning, motion execution, kinematics | Visualization of data, robot models, sensor outputs |
| 3D Simulation | Yes (full 3D environment) | No (only motion paths and execution) | Yes (visualizes robot model and data in 3D) |
| Integration | Integrates with ROS for testing and simulation | Integrates with ROS for motion planning | Integrates with ROS for visualizing robot states |
| Physics Engine | Yes (includes collision, gravity, friction) | No (not a physics simulator) | No (visualization tool only) |

- Exercise 1: Setting Up MoveIt for OpenManipulator-X

Step 1: Launch OpenManipulator-X on the Gazebo simulator (Terminal #1)

ros2 launch open_manipulator_x_bringup gazebo.launch.py

Step 2: Launch the open_manipulator_controller in Moveit & Rviz (Terminal #2)

ros2 launch open_manipulator_x_moveit_config moveit_core.launch.py

Step 3: Set the goal state to random and click on the Plan button in RViz. The MoveIt framework will compute a motion plan. Then, click Execute to perform the movement on the robot, and observe the results.

Step 4: try two random positions and observe the results

Put screenshots of your work here (for ALL the steps):

- Exercise 2: Subscribing to Joint States

Step 1: Understanding the /joint_states Topic

The /joint_states topic provides real-time information about the robot’s joint positions, velocities, and efforts. The data is published as messages of type sensor_msgs/JointState.

Each JointState message contains:

- position: A list of joint angles (in radians) that represent the current position of each joint.

- velocity: The rate of change of the joint positions (in radians per second).

- effort: The force or torque applied to the joints (in Newton-meters).

In this exercise, you will write a simple ROS 2 node to subscribe to the /joint_states topic and log the joint positions in real-time.

Task: You'll create a new workspace with your name (e.g., name_lab7_ws), create a package, and the write the following python script, then run it using ros2 command (please follow the same steps from previous lab to execute the code)

- Start Gazebo and MoveIt:

Launch the simulation environment and MoveIt to enable motion planning and control.

- Run the Subscriber Node:

In a separate terminal, execute your subscriber node to start listening to the /joint_states topic.

- Move the Robot using MoveIt:

Use the Plan & Execute feature in MoveIt to command the robot to a new position.

- Observe the Subscriber Output:

Monitor the terminal running the subscriber node to see the received joint states.

Put screenshots of your work here (for ALL the steps)::

- Lab activity:

Task1: Modify the Subscriber to Log Velocities and Efforts:

- Modify the code to log additional joint information like velocity and effort by accessing msg.velocity and msg.effort.

- Log both position, velocity, and effort in the callback function.

- Provide the new added lines in the code:

import rclpy

from rclpy.node import Node

from sensor_msgs.msg import JointState

import time

class JointStateSubscriber(Node):

def __init__(self):

super().__init__('joint_state_subscriber')

# Create a subscriber to the /joint_states topic

self.subscription = self.create_subscription(

JointState,

'/joint_states',

self.joint_states_callback,

10

)

self.get_logger().info("Subscribed to /joint_states")

def joint_states_callback(self, msg):

self.get_logger().info(f"Received joint states:")

self.get_logger().info(f"  Position: {msg.position}")

self.get_logger().info(f"  Velocity: {msg.velocity}")

self.get_logger().info(f"  Effort: {msg.effort}")

time.sleep(2)

def main(args=None):

rclpy.init(args=args)

node = JointStateSubscriber()

rclpy.spin(node)

node.destroy_node()

rclpy.shutdown()

if __name__ == '__main__':

main()

- Explain the role of each added line.

self.get_logger().info(f" Velocity: {msg.velocity}")

- This line logs the velocity of each joint.

- The velocity is measured in radians per second (rad/s) for rotational joints and meters per second (m/s) for prismatic joints.

- It helps in monitoring how fast each joint is moving in real-time.

self.get_logger().info(f" Effort: {msg.effort}")

- This line logs the effort (torque or force) applied to each joint.

- The effort is measured in Newton-meters (Nm) for rotational joints and Newtons (N) for prismatic joints.

- It is useful for analyzing how much force is being exerted on each joint, which can be important for torque-based control or detecting mechanical issues.

3. Provide screenshot of the terminal that’s running the code, and the OpenManipulator-X gazebo window and moveit:

References

https://emanual.robotis.com/docs/en/platform/openmanipulator_x/overview/