AI 407 – Introduction to Robotics

Lab 4 Manual

Exploring Robotics Fundamentals with Turtlesim

- Lab objectives

- Introduction to Turtlesim

- Exploring ROS 2 commands with Turtlesim

- Controlling Turtlesim with Python

- Lab Requirements

- Software: Ubuntu 22.04 LTS, ROS 2 Humble

- Hardware: Students should work on Lab Devices

- Before You Start

Kindly read the manual, review the references if any, before beginning implementation.

- Introduction to Turtlesim

Turtlesim is a lightweight 2D simulator included in ROS 2, designed for learning robotics and ROS fundamentals. It provides a simple turtle-like robot that moves in response to commands, making it ideal for beginners to explore ROS concepts like topics, services, and parameters.

Key learning aspects of Turtlesim:

- ROS Topics & Messaging: Users control the turtle by publishing velocity commands to the topic /turtle1/cmd_vel and monitor its position via the topic /turtle1/pose, introducing the publish/subscribe model.

- Basic Robotics Principles: Experimenting with linear and angular velocity helps understand motion and forward kinematics.

- Control Algorithms: Users can implement ROS nodes to move the turtle in patterns like circles or squares, gaining hands-on experience with robot control logic.

- Exercise 1: Exploring Turtlesim

Step 1: Source ROS 2 Environment

Before running any ROS 2 commands, source the setup file:

source /opt/ros/humble/setup.bash

Step 2: Run the Turtlesim node

ros2 run turtlesim turtlesim_node

Step 3: Understanding ROS 2 Topics for Turtlesim

Key topics for controlling the turtle:

- /turtle1/cmd_vel → Publishes velocity commands (geometry_msgs/msg/Twist).

- /turtle1/pose → Provides the turtle’s current position and orientation (turtlesim/msg/Pose).

  - To list all active topics:

ros2 topic list

  - Check the Message Type:

ros2 topic info /turtle1/cmd_vel

This will show you the type of the message being used (which is geometry_msgs/msg/Twist).

  - Check the Message Structure:

ros2 interface show geometry_msgs/msg/Twist

This command displays the structure of the Twist message, which consists of linear and angular components (both of type Vector3).

Vector3 linear:

float64 x        # Movement along the x-axis

float64 y        # Movement along the y-axis

float64 z        # Movement along the z-axis

Vector3 angular:

float64 x        # Rotation around the x-axis

float64 y        # Rotation around the y-axis

float64 z        # Rotation around the z-axis

Step 4: Controlling Turtle Movement

To move the turtle, publish a message to /turtle1/cmd_vel. The message type is geometry_msgs/msg/Twist, which contains linear and angular velocity components.

- Controlling the Turtle Using the Keyboard:

To control the turtle using the keyboard, run the following command in a new terminal:

ros2 run turtlesim turtle_teleop_key

- This command starts a teleoperation node that listens for keyboard inputs and publishes velocity commands to /turtle1/cmd_vel.

- Use the arrow keys, or the keys written in the terminal to move the turtle:

- Publishing Velocity Commands Manually

  - Example: Move the Turtle Forward

In a new terminal, publish a velocity command:

ros2 topic pub /turtle1/cmd_vel geometry_msgs/msg/Twist ‘{linear: {x: 2.0, y: 0.0, z: 0.0}, angular: {x: 0.0, y: 0.0, z: 0.0}}’ --once

This makes the turtle move forward with a speed of 2.0 m/s.

  - Example: Rotate the Turtle

To rotate the turtle to the left, publish a message with a positive angular velocity in the z-axis:

ros2 topic pub /turtle1/cmd_vel geometry_msgs/msg/Twist '{linear: {x: 0.0, y: 0.0, z: 0.0}, angular: {x: 0.0, y: 0.0, z: 1.5}}' --once

This makes the turtle rotate counterclockwise at an angular velocity of 1.5 rad/s.

Step 5: Monitor Turtle Pose

To track the turtle’s position & orientation in real time, subscribe to /turtle1/pose topic using:

ros2 topic echo /turtle1/pose

This command displays the turtle’s x, y position, orientation, and velocity as it moves.

Put screenshots of your work here:

- Exercise 2: Controlling Turtlesim with Python

In this exercise, you'll create a ROS 2 package to control the turtle using a Python script. You'll learn how to publish velocity commands programmatically instead of using terminal commands.

Step 1: Create a New Workspace

First, create a new workspace with your name (e.g., name_lab4_ws):

mkdir -p ~/name_lab4_ws/src

Step 2: Create a New Package

First, navigate to your ROS 2 workspace (e.g., name_lab4_ws) and create a package:

cd ~/name_lab4_ws/src

ros2 pkg create --build-type ament_python my_turtle_pkg --dependencies rclpy geometry_msgs

This creates a package named my_turtle_pkg inside the src folder.

Step 3: Write the Python Script

Navigate to the package directory and create a move_turtle.py script:

cd ~/name_lab4_ws/src/my_turtle_pkg/my_turtle_pkg

gedit move_turtle.py

- This script publishes velocity commands to /turtle1/cmd_vel every 1 second.

- The turtle moves forward (linear.x = 1.0) and rotates left (angular.z = 1.0).

- The node logs the published values for debugging.

After you write the code, make it executable

chmod +x move_turtle.py

Step 4: Modify setup.py to Include the Script

Open setup.py inside my_turtle_pkg and add the entry point:

'move_turtle = my_turtle_pkg.move_turtle:main',

Step 5: Build the Package

- Return to your workspace root and build the package:

cd ~/name_lab4_ws

colcon build

- Source the workspace in the. bashrc file:

source ~/name_lab4_ws/install/setup.bash

Step 6: Run the Script

First Run the turtlesim node

ros2 run turtlesim turtlesim_node

Then, in another terminal, run your Python script:

ros2 run my_turtle_pkg move_turtle

Put screenshots of your work here:

- Lab activity:

Task: Write a Python script to move the turtle in a square path like this:

- Provide the source code:

import rclpy

from rclpy.node import Node

from geometry_msgs.msg import Twist

import time

class TurtleController(Node):

def __init__(self):

super().__init__('turtle_controller')

self.publisher = self.create_publisher(Twist, 'turtle1/cmd_vel', 10)

self.timer = self.create_timer(1.0, self.move_turtle)

def move_turtle(self):

msg = Twist()

msg.linear.x = 1.0  # Move forward

msg.angular.z = 0.0  # Rotate left

self.publisher.publish(msg)

self.get_logger().info(f'Publishing linear x={msg.linear.x}, angular z={msg.angular.z}')

time.sleep(1)

msg.linear.x = 0.0

msg.angular.z = 1.57  # Rotate left

self.publisher.publish(msg)

time.sleep(1)  # Correct indentation

def main(args=None):

rclpy.init(args=args)

node = TurtleController()

rclpy.spin(node)

node.destroy_node()

rclpy.shutdown()

if __name__ == '__main__':

main()

- Explain what you did to make it move in square shape:

Just move forward and set the rotation to 0 and sleep so it will make a line

Then set the line to be 0 and only rotate by 90 degrees

3. Provide screenshot of the terminal that’s running the code and the Turtlesim window:

References

Creating a workspace — ROS 2 Documentation: Humble documentation